Full Specs for 1979 CM400a

Dimensions

Overall Length			83.1 in

Overall Width				34.3 in

Overall Height				45.3 in

Wheel Base				56.1 in

Seat Height				31.5 in

Foot Peg Height			12.2 in

Ground Clearance			  5.5 in

Dry Weight				392.4 lbs

Frame

Type					Diamond

Front Suspension/Travel		Telescopic Fork, 139.5 mm

Rear Suspension/Travel		Swing Arm, 75.9 mm

Front Tire Siz				350S18 4PR

Rear Tire Size				460S16 4PR

Front Brake				Disk Brake

Rear Brake				Internal Expanding

Fuel Capacity				2.6 US gallons

Reserve Capacity			0.8 US gallons

Caster angle				60*30” from horizontal

Trail Length				4.3 in

Front Fork Capacity			4.9 oz

Engine

Type					Air Cooled, 4 stroke OHC

Cylinder Arrangement		Vertical Twin Parallel

Bore and Stroke			2.776 x 1.992 in

Displacement				24.1 cu in

Compression Ratio			9.3:1

Valve Train				Chain Driven Overhead Camshaft

Oil Capacity				3.5 US quarts

Lubrication System			Forced Pressure and wet sump

Cylinder head compression pressure	185 + - 14psi

Valve Clearance			.004 intake,  .006 exhaust

Carburetor

Type					CV type, 28mm, venture bore

Drive Train

Transmission			2 speed semi automatic with torque converter

Electrical

Ignition				Capacitive discharge ignition