Vectrex Development System '95

------------------------------

The Hardware

------------

The system basically consists of a ram chip, two eproms and a serial interface.

A counter is used to switch between 32k ram and the various eprom banks which

have a size of 16k each. As the cartridge address area is limited to 32k,

the memory map looks as follows:

Counter

Value       0-7                 8-15

------------------------------------

$0000-$1fff eprom bank 0-7 (*)  RAM

$2000-$37ff not used            RAM

$3800-$3bff counter strobe(**)  counter strobe

$3c00-$3fff ACIA registers      ACIA registers

(*)  minimum configuration is one 2764 eprom ->4k

max config. two 27512 eproms ->128k

(**) each read/write access increases the counter by one

A serial cable (RS232) can be used to connect the devsys with any terminal.

The Software

------------

Currently a modified version of vmon (by Jeff Lavin) is used as bootstrap.

It allows you to examine memory/registers and to download hexcode on the

development system ram. The sourcecode has been modified for assembling with

the as09-package.

Future versions will (hopefully) feature a nice 3d-menu allowing

to choose between boot-monitor, rom-programs and minestorm.

The Layout

----------

I actually made a layout for the devsys, but it turned out to have some bugs

which I had to correct manually during testing. If there are some people

interested, I will consider to release a corrected version.

Useful Hints

------------

- try checking ALL solder bridges, even if they look allright

- remember to CROSSconnect the RS232 handshake lines

Instructions

------------

The following commands are usable for the boot monitor:

m	Memory exam (read/write)

g	Go (set Program Counter)

r	Register exam

v	Verify Memory

t	Transfer file

p	Protocol change

memory exam / modify

--------------------

m <EA><CR>   -> '<CR><EA>,<CONTENTS>,'

next expected inputs are:

a) HEX numbers ->these are stored as of <EA>, and the former

contents are printed on screen.

b) '<' ('-')   -> go back 1 (16) Bytes

c) '+'         -> forward 16 Bytes

d) '!<ASCII>   -> store <ASCII>+$80 in <EA>

e) <ANY KEY>   -> show contents of next address

f) <CR>        -> end examine mode

<EA> = address (e.g. $4ac0)

<CR> = carriage return

'go' command

------------

'g <EA>' -> go to <EA>

'g'      -> go to <PC>

verify memory

-------------

no comment

(I didn't ever use this, so you'll have to find out for yourself...)

protocol change

---------------

'p:1'+<CR>	-> 2400  Baud

'p:2'+<CR>	-> 9600  Baud

'p:3'+<CR>	-> 19200 Baud

default setting is 9600 Baud

transfer file to vectrex

------------------------

't <EA>'+<CR> -> load to <EA>

't'+<CR>      -> load to $0000

next expected inputs are (use your terminal program's 'ASCII UPLOAD'):

<hexbyte>-> is written to memory at current location

<CR>	   -> ends transfer

<hexbyte> = two-digit ASCII format number (e.g. 7f)

please note:

during transfer the vectrex screen is switched off

(software bug: my transfer protocol doesn't use RTS/CTS)

----------------------------------------------------------------

send comments & suggestions to:

ub2f@rz.uni-karlsruhe.de (Ralph Kimmlingen)